# DES-3500 Series Firmware Release Note

Firmware: 5.00.B27

Hardware: A1/A2/A3/A4G

Date: Oct.1, 2007

New features :

- IP-MAC-Port Binding v2.5

- Ability to set terminal length

- LBD ( Loopback Detection ) v4.0

- Add "Delete ACL all" CLI command

- DHCP Client Filtering

- DHCP Server Screening

- NetBIOS filtering

- Log message for those shutdown ports caused by broadcast storm

- Ability to display tagged ports One more line in CLI command "Show vlan" so it can, currently we need to use all "member ports" minus "untagged ports" to get the results

- Remove the limitation when browse MAC of MAC table in Web UI, user must key in MAC address with "-", for example "12-23-34-45-56-67"

- Expand per port limit IP multicast address range from 10 to 30 / per device max 256 entries.

- " Auto recovery " on blocked port caused by IP-MAC-Port binding access violation

- 802.1D v2004

- Remove all trust host by one command in CLI, Web and SNMP

- Extend trust host number to 10

- Add subnet mask parameter on trusted host, for the ability to input not only one IP for trusted host

- Host-based IGMP snooping

- Add a SNMP MIB to support VLAN advertisement enable/disable function

- Auto finish the commands which will pop up (Y/N) message with and extra parameter force_agree

- Add a switch to enable/disable IP 0.0.0.0 checking on IP-MAC-Port binding ARP mode

- 802.1x Guest VLAN

- ISM VLAN Enhancement: The source address of the IGMP join/leave from the different hosts/subnet will be replaced with the pseudo IP address of the joining/leaving ISM VLAN before sent from server port.

- Display remaining number of ACL rules on Web/CLI

- Add ping command to user privilege

- Stop contacting to TACAS+ server periodically before logging-in

- Three-Level User Account

- Recovery timer for broadcast storm shutdown port ( 5 minutes, fixed )

- Password Recovery v1.1

- Rename " DLF " to " Unicast " in storm control feature

- Remove the login screen on WEB

- Add new command show acl config for user privilege

- Change Download/Upload commands message

- Change the hyperlink address of WEB’s DLINK LOGO. (from www.dlink.com to www.dlink.com.tw)

- Change the behavior of “enable admin” command on RADIUS and local authentication: Under the structure of “ Three-level user account”, if the user is authenticated by RADIUS or Local, this user can not change its privilege to administrator level with “ enable admin” command

- Support per flow bandwidth control by ACL

- Add “Limited IP Multicast ranges” web page into Web UI.

- The GVRP feature distributes the VLAN information to the networks even that VLAN does not have any link up port

Problem Resolved :

1. DES-3526 enters exception mode while user connect to the device with SSH script.

2. Wrong NAS Port ID display on some ports in 802.1x authentication message.

3. “Forbidden router port” setting is missing after the device reboot.

4. The device enters exception mode while creating additional ACL Profile.

5. Can not create ACL Ethernet profile by using SNMPSet command.

6. Device keeps sending port security violation traps even the MAC has been learned in FDB table.

7. When type the password in the CLI interface, user should only see “*” instead of the characters.  But if user move the cursor to the first “*” and delete it, the reset of “*” will reveal to the normal characters. (password will become plain text, not “*”).

8. Fail to use TFTP to upload/download configuration or firmware from WEB interface while uses IP address 10.255.10.5 and 10.255.255.1

9. Fail to close the browser windows (IE and Firefox) from WEB interface if user type the wrong user name and password.

10. If open the SIM member’s web from commander’s SIM web page, incorrectly two duplicated browser windows will be opened.

11. When configure the MSTP edge port as “True” via WEB, the device will show “False” at CLI.  When user configures the MSTP edge port as “False” via WEB, the device will show “True” at CLI.

12. DES-3500 CLI incorrectly responds the success message for incompleted “safeguard engine” command.

13. The CLI command “show ports” will have wrong display when user executes “config ports all trap disable”. User will see there is a letter “d” in the right column of the screen.

14. When the device enters the exhausted mode, it can not go back to normal mode even the CPU utilization becomes normal.

15. When delete the IP-MAC-Port binding entry, the device only check the IP address as the index. If delete the correct IP with incorrect MAC, the entry remains removed successfully.

16. When the additional instance id is created for the MSTP, there will be influence on the original working MSTP instance.

17. If the port has been set with bandwidth control parameter, it can not be set as the LACP member port.

18. Change the configuration line feed from 0D 0A to 0A 0D.

19. The cold start trap has the wrong source IP address in the packet content.

20. Configuration of the edge port in STP can not take effect after user save and reboot the device.

21. Slow display of telnet client function

22. RSTP has long convergence time when topology event happens.

23. When the “port security trap” is enabled and access violation happens, the system hangs up.

24. The device enters exception mode when user executes “reset system” via CLI.

25. User account has the same privilege as the Administrator, user can delete/create the IP-MAC-Port binding setting.

26. High CPU utilization even there’re few ARP packets received while the IP-MAC-Port binding is enabled.

27. Change the management traffic(telnet) priority sent out from the switch itself from normal priority 0 to high priority 7

28. Can not identify the model name of DEM-310GT.

29. Change the PVID to 1 if the port is removed from default VLAN (Default PVID is 0 if the port is removed from default VLAN)

30. The CLI screen will be a mess when typing-in “show packet ports” and “show error ports” commands.

31. GVRP can not distribute the VLAN properly if there is a topology event happen when running RSTP.

32. The loop happens while running multicast traffic in the ring topology even with RSTP enabled.

33. When connecting to the DES-3526 with very old combo card (DE-528T), it only can connect at 10M Half duplex mode.

Firmware: 4.01.B19

Hardware: A1/A2/A3/A4G

Date: Jul. 21, 2006

Problems Resolved:

- Can not apply created ACL rule via Web interface.

- If configure port description contains more than 32 characters and use web browser to show it, the device will enter exception mode.

- Device will enter exception mode when user delete SNMP user and try to get the mib OID1.3.6.1.6.3.

- Fail to add Packet Content Mask if all "0" through SNMP

- Can not display the long log messages completely.

- Unable to save static ARP entries

- The IfOctets and IfInOctets obtain the counter value from the same stastistics source.

- IGMP_Snooping Router Time Out mechanism does not work correctly.

- Can not leave IGMP Snooping group properly if there are more than one client in the same group.

- Dynamic router port can not forward multicast data.

- The "zoom in" and " zoom out" functions were opposite from SIM topology.

- Correct typo "Enalbed" to “Enable” in Multicast router only setting.

- Can not create the Access profile successfully via D-View.

- Can not get parameter from some Bridge MIB tables via D-view 5.1.

- Enabled igmp_snooping mutlicast configuration becomes disabled after rebooting

- Port-based 802.1X problem: port is authorized, but ping failed.

- Default VLAN can be deleted via web interface

- Log bugs with RPS DPS-200.

- Long convergency time when using 18 pcs devices to test RSTP function.

New features :

- Password recovery

- CPU Interface filtering

- SIM R1.6

- IP MAC Port binding  ACL mode

- Log enhancement

- Telnet Client

- Broadcast storm control granularity down to 1 pps

- SIM base on system VLAN, , previously SIM only works on default VLAN

- Editable login banner

- Send trap in SNMP /Add warning message in syslog while access violation happens for IP MAC Port binding and port security

- Initial Caps on the login

- Disable the page mode in CLI

- Enhanced Broadcast storm control ( port shutdown )

- Forbid MC router or disable dynamic MC router

- Safeguard engine includes broadcast storm filtering

- Disable login banner

- Hot-key to delete all the characters behind the cursor in CLI.

- Max. port security learned address entries are enlarged from 20 to 64.

- Change web panel for new xStack outlook

=================================================================

Firmware: 3.06.B20

Hardware: A1/A2/A3

Date: Dec. 27, 2005

Problems Resolved:

Fix the security issues recently reported for unauthorized system access.

Enhancements:

Limit the access by passwords generated by "D-Link PWD caculator".

Before, if customers forgot their configured passwords, D-Link could generate passwords based on the MAC addresses provided by customers via D-Link HQ maintained PWD caculator. Thus customers can use those "backdoor" passwords via telnet/ web/ console to logon to their switch again.

Now the access of those passwords generated by "D-Link PWD caculator" will be limited to console access only.

This is to minimize the security concern.

Note:

This is a recommended security patch for your customers.

=================================================================

Firmware: 3.06.B09

Hardware: A1/A2/A3

Date: Nov. 21, 2005

Problems Resolved:

- Can not compile R3.5 MIB “dlkMSTP.mib”

- Can not show TFTP server IP correctly with MIB browser after downloading config file by SNMP.

- If setting TFTP server IP=0.0.0.0, Des3526 TFTP service hang-up. Need reboot to recover.

- Unable to delete "swACLIpRuleTable" by using SNMP

- Error message "System allocate memory fail!" appears while using "Show config" command in SNMP.

- Cannot add SNMP Host through Web interface

- RMON etherHistoryUtilization (1.3.6.1.2.1.16.2.2.1.15) of RMON always has the value "0",

- Can not create the ACL packet_content _mask through D-view 5.1

- Can not keep “packet_content_mask” setting after system reboot

- DES-3526 enters EXCEPTION MODE when configuring SNTP and ACL from Web.

- When DES-3526 flow control enabled port connects to an equipment which does not support flow control or disable flow control, the port still shows “100M/Full/802.3x”. The correct one should be “100M/Full/None”.

- DES-3526 Gigabit UTP port Speed link was not displayed in Web interface

- Can not set and display SIM group name through SNMP

Enhancements:

- IP Mac port binding

- STP loopback prevention

=================================================================

Firmware: 3.05.B09

Hardware: A1/A2

Date: Apr. 26, 2005

Problems Resolved:

- The same ARP entry can’t be added as static if it has been learned already.

- SSH connection can be formed by any IP even trusted host function is enabled.

- The setting of ACL tcp destination port isn't written to the configuration in WEB UI, but okay in CLI.

- The value of OID “dot1qTpFdbTable” is wrong after certain SNMP retrieving sequences.

- Configuration file cannot be restored after backup in PC. (This problem was found in R2.00B19, and already fixed in R3.01B28)

- Device crashed when enabling IGMP snooping and sending multicast traffic for a period of time.

Enhancements:

1. Can configure IP address 127.0.0.1 as system interface for SIM use.

2. Per port limit IP multicast address range

3. DHCP relay option 82

4. RFC2925 PING MIB & TRACEROUTE MIB

5. ICMP ping by MIB

6. Port Security expands to 20 entries

7. IP-Mac binding (512 entries)

8. Support new CLI command to apply one ACL rule to more than one port (the port range command in R2)

=================================================================

Firmware: 3.01.B28

Hardware: A2

Date: Mar. 25, 2005

Problems Resolved:

1. Clock time delays around 13 minutes a day.

2. From SNMP, the ACL mask containing Source IP Mask (or Dest. IP mask) cannot be created.

3 MSTP was found the issue while trying to implement 2 (or more) Spanning Trees on 2 (or more) physical links separately for load sharing and fault tolerance, only 1 physical link was active.

4. Cannot delete static fdb from telnet/cli (web OK).

5. Cannot use WEB login after operating a period of time.

6. MAC-based 802.1X WEB display of authenticated MAC error.

7. Security Risk on Member switch with SIM

8. Cannot add/remove member ports from default VLAN using MIB.

9. Entering debug mode 802.1x.

10. Improve SSL performance.

11. Exception dump when enable AsymVlan from Web (DUSA)

12. When upgraded from 1.xx, device (about 5% prob.) may dump exception.

13. Cannot configure ACL from web when select type=IP.

14. Rapid STP does not work, it acts like STP.

15. Solved the special version for CHT (3.01B23)

Enhancements:

1. Enlarge static VLAN to 255.

2. Add show config only command.

3. Catagorize 3 level of logs, and can send syslog & trap based on configured levels of events. (The main reason is to save flash and bandwidth)

4. Enlarge static ARP to 255

5. Enlarge ACL entry to 500

6. DHCP Boot

7. SSL Preload

8. Enlarge IGMP Snooping group from 64 to 128.

9. Per-port BPDU packet filtering.

10. “Autoconfig” function is added:
When autoconfig is enabled, the switch will be given a TFTP server address. The configuration file will be loaded upon booting up.

Notice: Because of ACL format change in R3 (per-port ACL), when upgrading from R2 firmware, ACL configuration will lose, and need to re-configure.

DES-3526 runtime code release history  (updated: Oct. 11, 2004)

Firmware 2.00.B19

Bug fixed.

1. Fixed SIM gets wrong copyright.

2. Fixed web gets wrong front panel.

3. Fixed port description "random characters" problem,

which may result in configuration lost problem after reboot.

4. Fixed the "Login Method List setting" in "Access Authentication Settings" from WebUI problem.

CLI/Telnet has no such problem.

New features:

None

DES-3526 runtime code release history  (updated: Aug. 30, 2004)

Firmware 2.00.B15

Bug fixed.

1. Fix system hang issue while changing 1x mode from PAP to EAP.

2. Fix DES-3526 packet loss when disable/enable no-connection port

3. Fix Port STP configurations lose after load default and save root.

a. Do not retrieve hello time parameter in 1d mode

b. Cost would be changed as auto while STP version changed.

4. IGMP snooping cannot disable via CLI command,

5. STP configurations instance lose after system reboot

6. Fix Giga port cannot forwarding frames in the first link up after boot up.

7. Fix stop forwarding packet after a period of running problem.

(for 6, 7, re-pluging any port can recover the symptom)

8. Fix web lacp page error message and incorrect lacp information¤

New features:

None

DES-3526 runtime code release history  (updated: Jun. 9, 2004)

Firmware 2.00.B08

Bug fixed.

1. Fixed Device hangup when port 80 Web attack.

New features:

1. Support SSH2

2. Support Radius Authentication on Switch Admin access

3. Support SSL.

4. Support 802.1s (MSTP, Multiple Spanning Tree Protocol)

=============================================================

DES-3526 runtime code release history  (updated: Apr.6, 2004)

Firmware 1.00.B17

Bug fixed.

1. SIM member device firmware download problem at 32 member environment.

2. SIM 32 members re-config problem

3. SIM device hanging up problem.

First Release

DES-3526 runtime code release history  (updated: Mar., 2004)

Firmware 1.00.B15

New features:

First release version.  For the features, please refer to the datasheet.

Bug fixed.

N/A

=======================================================================